# Floor Plan Standard – Instructions

# PURPOSE

This guide defines mandatory requirements for all floor plans created on behalf of Propus GmbH (6300 Zug, Switzerland).

It covers the calculation of the Swiss net living area (Nettowohnfläche), labeling conventions and layout requirements.

# SOURCE OF AREA VALUES

All area values (m²) and room labels are taken 1:1 from the original architect plan.

Authoritative sources in descending priority

Architect plan (BF values = Bodenfläche, light interior area)

On-site measurement data

Never estimate or extrapolate from rectangular dimensions

⚠️  If the dimension (e.g. 4.18 m × 5.47 m) does not match the architect plan area, the architect plan area is authoritative. The room may not be rectangular.

# NET LIVING AREA (NWF)

The net living area (Nettowohnfläche, NWF) is defined by the Swiss Federal Office for Housing (BWO) and corresponds to the Net Floor Area (NGF) in SIA standard 416.

Included in NWF

All heated, year-round habitable rooms inside the dwelling

Living room, bedrooms, kitchen, bath/WC, hallway, vestibule, corridor

Stairs within the same dwelling (e.g. internal stairs in a single-family house between UG/EG/OG/DG)

Walk-in closets (Umkleide), built-in wardrobes

Heated storage rooms inside the dwelling

Floor area under sinks, toilets, kitchen counters

Minimum room height 1.50 m (under sloped roofs: only floor area where ceiling ≥1.50 m)

NOT included in NWF

Wall cross-sections (interior and exterior walls) — NWF is wall-to-wall, light interior area only

Balconies, terraces, loggias, unheated winter gardens → Aussennutzfläche (outdoor area)

Cellars, attics (Estrich), garages, hobby rooms, laundry rooms → Nebennutzfläche (ancillary)

Unheated rooms

Common stairwells in multi-family buildings (Verkehrsfläche) — only stairs that belong exclusively to the dwelling count

Structural formula

Nutzfläche (Total Usable Area) = Nettowohnfläche (NWF) + Nebennutzfläche (cellar, attic, hobby room, laundry, garage) + Aussennutzfläche (terrace, balcony, patio, loggia)

Always show separately on the floor plan

NETTO-WOHNFLÄCHE: 291 m²

NUTZFLÄCHE: 413 m² (incl. Patio: 18 m², Terrasse: 23 m², Auffahrt: 44 m², Balkon: 37 m²)

# LABELING RULES

All labels must be in German, following Swiss real estate convention. English abbreviations or mixed terms are not permitted.

Rooms – always label as

Umkleide (NEVER W.I.C, WIC, Walk-in Closet, Ankleide)

Schlafzimmer (NEVER Bedroom, BR)

Wohnzimmer (NEVER Living Room, LR)

Küche, Esszimmer / Essen, Büro

Vorraum (within the dwelling), Gang (passage / circulation)

Eingangsbereich or Mud Room (NEVER Entry, Foyer); Entrée

Bad (full bathroom with bathtub); DU/WC (shower + toilet); WC (toilet only)

Reduit or Abstellraum (NEVER Storage)

Technikraum (NEVER Utility, Mechanical)

Keller (NEVER Cellar, Basement)

Garage (Carport only if open)

Terrasse (ground floor or DG); Balkon (cantilevered upper floor); Patio (enclosed inner courtyard, ground level)

Auffahrt (NEVER Driveway)

Floor labels

Untergeschoss (UG) — basement / lower level

Erdgeschoss (EG) — ground floor

Obergeschoss (OG) — upper floor

Dachgeschoss (DG) — top / attic floor

⚠️  Floor designation must always match the architect plan. If the architect labels the top floor as „Dachgeschoss", we also use „Dachgeschoss" — not „Obergeschoss".

# LAYOUT STANDARD

Format: A3 or A4 landscape, depending on complexity

Resolution: 300 dpi for PDF export

Font: sans-serif, well-readable (Helvetica, Inter, Open Sans)

Room label font size: minimum 10 pt

Dimension font size: 7–8 pt

Entrance arrow with label „EINGANG"

North arrow when orientation is not obvious

Furniture visible: bed, sofa, kitchen appliances, WC, sink, bathtub, shower

Room caption (per room)

Label (e.g. „Schlafzimmer")

Dimension in format 4.18m × 5.47m

Area in m² per architect plan (e.g. 18.2 m²)

Footer at the bottom of each plan

Full address: street name, postal code, city, canton

Floor designation

Total areas (NETTO-WOHNFLÄCHE / NUTZFLÄCHE)

Disclaimer: GRÖSSEN UND ABMESSUNGEN SIND UNGEFÄHR, DIE TATSÄCHLICHEN KÖNNEN ABWEICHEN.

# TEXTURES AND PERFORMANCE

Textures such as natural stone, parquet, lawn, or paving must be embedded as rasterized images (PNG/TIFF, approx. 300 dpi).

⚠️  NEVER embed as vector patterns. Vector textures generate huge Form XObjects that crash Adobe Acrobat as soon as someone tries to edit the PDF later.

# PRE-DELIVERY CHECKLIST

Before sending the final PDFs, verify

All rooms from the architect plan are shown on the floor plan (including Mud Room, Entrée, Reduit)

Room labels follow the standard (no English abbreviations like W.I.C)

Floor designation matches the architect plan

Per-room areas match the architect plan BF values (not estimated from dimensions)

Total NWF, Nutzfläche, and Aussennutzfläche listed separately

Stairs correctly classified: SFH stairs count toward NWF; MFH common stairwells excluded

Terrace / balcony / patio correctly named and listed under Aussennutzfläche

Disclaimer present in the footer

Textures embedded as rasterized images, not vector patterns

# CONTACT

Questions: office@propus.ch

Version 1.0 · Propus GmbH